FORMER CIRCLE K 1461

WASTE DISCHARGE AUTHORIZATION No. 4614-01

MONTHLY SYSEM WATER SAMPLING RESULTS

| Sample Location | Sample Date | Nonpolar FOG by EPA Method 1664B | Chemical of Concern |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VOCs by EPA Method 8260D |  |  |  |  |  |  |  |  |  |
|  |  | Nonpolar FOG | Benzene | Toluene | Ethylbenzene | Total Xylenes | TCE | Cis-1,2-DCE | PCE | Trans-1-2-DCE | Vinyl Chloride |  |
| Screening Level1 (mg/L): |  | 100 | 0.070 | 1.4 | 1.7 | 2.2 | 0.50 | 1.0 | 0.24 | 1.0 | 0.012 |  |
| Untreated Influent, prior to GAC vessels | 12/18/2024 | 36.1 | 0.00851 | 0.00169 | 0.001 U | 0.0273 | -- | -- | -- | -- | -- |  |
|  | 1/17/2025 | 5.88 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | -- | -- | -- | -- | -- |  |
|  | 02/27/2025 | 11.9 J- | 0.00209 | 0.00183 | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.00336 | 0.001 U | 0.001 U |  |
|  | 03/21/2025 | 6.31 | 0.001 U | 0.001 U | 0.001 U | 0.00604 | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 04/25/2025 | 63.9 | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.0159 | 0.001 U | 0.001 U |  |
|  | 5/23/2025 | 6.1 | 0.001 U | 0.001 U | 0.001 U | 0.00794 | 0.00125 | 0.00120 | 0.0160 | 0.001 U | 0.001 U |  |
| Midpoint between GAC vessels | 12/18/2024 | 6.63 | 0.001 U | 0.001 U | 0.001 U | 0.003 U | -- | -- | -- | -- | -- |  |
|  | 1/17/2025 | 5.88 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | -- | -- | -- | -- | -- |  |
|  | 02/27/2025 | 5.62 UJ | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 03/21/2025 | 5.81 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 04/25/2025 | 5.80 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 5/23/2025 | 6.7 | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
| Treated effluent, post GAC vessels | 12/18/2024 | 5.05 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 1/17/2025 | 5.49 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 02/27/2025 | 5.88 UJ | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 03/21/2025 | 5.95 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 04/252025 | 6.00 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |
|  | 5/23/2025 | 6.00 U | 0.001 U | 0.001 U | 0.001 U | 0.003 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U | 0.001 U |  |

Notes:

1 = Screening levels are based on effluent limits in the KCIW Discharge Permit No. 4614-01.

Bold = sample result was detected; Yellow = sample result exceeds the screening level

DCE = dichloroethene

EPA = U.S. Environmental Protection Agency

FOG = fats, oils, and grease

GRO = gasoline-range organics

KCIW = King County Industrial Waste

mg/L = milligrams per liter

PCE = tetrachloroethene

TCE = trichloroethene

VOCs = volatile organic compounds

-- = sample was not analyzed for analyte, and/or chemical of concern

Laboratory Qualifiers:

J = Estimated:  The analyte was positively identified; the quantitation is an estimation.

J- = Estimated, Low Bias:  The result was an estimated quantity, but the result may be biased low.

U = Not detected at the limit of detection.
UJ = Not detected at the limit of detection, but the limit is an estimation.